# Chapter 3 — Networking Connectors & Wiring Standards:

## Coaxial Cable:

Coax is a single copper center conductor, wrapped in a plastic insolation layer, with a braided shield over that, then an outer jacket of PVC or a Teflon-type plenum-rated coating. It's big advantage: the braided shielding resists EMI/RFI (electromagnatic/radio frequency interference) and other stray signals.

Plenum vs. non-plenum: the plenum is the space between floors in multistory buildings used to circulate air, and often carries cabling too. Fire code (NFPA, National Fire Protection Association) requires plenum-rated cable there, because regular (non-plenum) insulation gives off poisonous smoke and can help fire spread like a wick. Non-plenum is cheaper, so it's fine anywhere that isn't a plenum space.

| Type | Standard | Cable | Size | Connector | Status |
| --- | --- | --- | --- | --- | --- |
| Thinnet | 10Base2 | RG-58 | ~5mm | BNC with 50-ohm terminating resisters on both ends | Essentially obsolete for LANs |
| Thicknet | 10Base5 | RG-8 | ~1/2 inch | Worked the same way | Basically extinct |
| Cable TV cable/modem |  | RG-59 Or RG-6 |  | F-type screw-on connector | Where coax survives today. |

## Twisted-Pair: UTP vs. STP, and Categories:

Twisted-pair is multiple individually insulated wires twisted together in pairs. The twists are there specifically to reduce cross-talk (interference from nearby signals) and protect against outside interference.

- UTP (Unshielded Twisted Pair): What you'll touch every day. Used in Ethernet standards like 10BaseT through 40GBaseT. It wins today because it's cheap, easy to work with,  and support speeds that weren't possible a decade ago.

- STP (Shielded Twisted Pair): adds a metal foil or braid around each pair for better noise immunity in electrically noisy environments.

### Categories

Each higher category is rated for a higher frequency and has more twists per inch.

| Category | Frequency | Speed/use | Status |
| --- | --- | --- | --- |
| Cat 1 | 1 MHz | Voice only (POTS, Plain Old Telephone Service). Not rated for dat | Voice only |
| Cat 2 | 10 MHz | 4 Mbps | Obsolete |
| Cat 3 | 16 MHz | 10 Mbps Ethernet | Obsolete for networks |
| Cat 4 | 20 MHz |  | Obsolete |
| Cat 5 | 100 MHz | 100BaseTX (two-pair) | Largely replaced by Cat 5e |
| Cat 5e | 100 MHz | Gigabit. Handles crosstalk from all four pairs transmitting at once. | Practical minimum or any modern install |
| Cat 6 | 250 MHz | Gigabit comfortably. Standard since June 2002 | Go-to for new buildings. Common as riser cable between floors, alongside fiber |
| Cat 6A | 500 MHz | Better crosstalk handling lets 10GBaseT run the full 100m (regular Cat 6 has a shorter max distance at that speed) | Current |
| Cat 7 |  | 10 Gigabit Ethernet over 100m of copper | Current |
| Cat 8 |  | 25G/40G at 30m | Built for data centers |

### Practical Tips

- Yanking Cat 5e stretches its internal twists and can invalidate its rating. Cat 6 has a plastic spine to resist that, and Cat 7/8 add metal shielding.

- Always test and connect all four pairs, even though older setups used only two. Gigabit needs all eight wires in good shape.

- A true Cat 5e/6/7/8 install means every component end-to-end (patch cables, wall panel, patch panel) meets spec. One mismatched piece breaks the whole certification, and verifying it takes pricey test gear.

## RJ Connectors:

BNC connectors don't fit UTP, so RJ (registered jack) connectors are used instead. They're attached with a crimper, just with a different die shape than BNC.

| Connector | Positions / Wires | Used For | Notes |
| --- | --- | --- | --- |
| RJ-11 | Small 4/6-position jack. Four wires (two pairs) | Phone lines and home DSL (Digital Subscriber Line) | Never used for LANs |
| RJ-45 | 8-position jack. Eight wires (four pairs) | Ethernet data | Unshielded wiring |
| RJ-48c | Looks identical to the RJ-45 | T1 circuits (long-distance WAN), not short LAN runs | Wired differently, and the wires are typically shielded. Don't mix them up by sight alone. |

## Straight-Through, Crossover, Rolled Cable & T568A/B:

### T568A  vs. 1568B:

Over 60 vendors agreed on two wiring standards, T568A and T568B, though not universally, so you'll find both in the wild. The entire difference: pair 2 (orange) and pair 3 (green) swap positions.

- T568A: green on pins 1—2,orange split accross pins 3 and 6.

- T568B: those two pairs swapped (orange on 1—2, green one on 3 and 6).

- Pins 4,5,7,8 (blue and brown pairs) never move. They're needed for 1000 Base T, PoE (Power over Ethernet), and some 100 Mbps setups.

- Since VoIP (Voice over IP) commonly shares the same wire as data today, new installs should always collect and use all eight pins.

### Three Cable Types:

| Cable | Connects | Wiring |
| --- | --- | --- |
| Straight-through | Unlike devices: host to switch/hub, or router to switch/hub | Same standard on both ends (T568A + T568A, or T568B + T568B). For basic 10/100 Ethernet only pins 1, 2, 3, 6 are used, wired 1-to-1, 2-to-2, and so on. A Gigabit straight-through is still 1-to-1 through 8-to-8 |
| Crossover | Like devices: switch-switch, hub-hub, host-host, hub-switch, or router direct to host. | T568A on the one end, T568B on the other. Pin 1 goes to pin 3 and pin 2 to pin 6, on one end only |
| Rolled / rollover | A host's serial interface to a Cisco router or switch console port, for management access | Carries no Ethernet at all. Easiest to make: flip the connector on one end of a straight-through cable and reattach it |

For Gigabit (1000BaseT), all four pairs (all 8 pins) are used simultaneously, not just two pairs.

### Crossover :

- Always label a crossover cable clearly so no one mistakes it for a regular patch cable. Plugged in wrong, the workstation can't talk to the network.

- A cable tester can confirm which type you're holding, or flag a bad cable.

- Handy trick:  carry a crossover cable to test a server's NIC (Network Interface Card) directly from your laptop without going through a switch.

T1 crossover cable: niche, used only to connect to CSU/DSU (Channel Service Unit/Data Service Unit) devices back-to-back. It swaps pins 1/2 with pins 4/5 rather than using the Ethernet crossover point.

## Fiber-Optic Cable & Connectors:

Fiber transmits digital signals as light rather than electricity, which makes it immunebu EMI/RFI.

Construction, inside out: a glass or plastic core (glass carries father, plastic is cheaper), then cladding that reflects light back into the core, then a flexible buffer, sometime Kevlar armour, then a PVC or plenum jacket.

- Pros: totally immune to EMI/RFI, and can run up to ~40 km C

- Cons: harder to install, more expensive (cable and test equipment), and harder to troubleshoot.

### Single-Mode vs. Multimode:

|  | SMF (Single-Mode Fiber) | MMF (Multimode Fiber) |
| --- | --- | --- |
| Light | One laser-driven ray (one or two glass strands, with LED or laser sources) | Multiple LED-driven rays, dispersed across multiple paths and reflected back by the cladding |
| Distance | Long runs. Up to 90x farther than multimode, 40-80km depending on the transceiver | Medium runs, up to ~3000ft. Gets unreliable beyond that |
| Speed | Faster | Great with bandwidth over medium distances |
| Cost / install | Trickier. Shouldn't be crimped or bent sharply | Cheaper. Comes in glass or an easier-to-install plastic version. |
| Typical use | Between buildings | Within one building |

### Connectors

| Connector | What it is | Notes |
| --- | --- | --- |
| ST (straight tip) | AT&T designed, bayonet twist-lock (like BNC) | Popular for SMFWAN links. won't loosen over time. |
| FC | Similar to ST, but with a screw mechanism | less common |
| SC | Square, push-on/off. Nicknamed “Square Charlie” | Popular in MMF installs, but larger and being phased out for smaller connectors |
| MT-RJ | SSF (small form factor). One-third the size of SC/ST, combines TX/RX in one connector | keyed for single polarity. No polishing or epoxy needed |
| LC | SFF. Resembles an RJ connector with a spring-loaded detent. | Smaller for higher port density. Popular for Fibre Channel and Gigabit Ethernet. Can't be disassembled to swap TX/RX like SC can |
| MPO | High-density SFF connector, fitting 2,8,12, or 24 fibers in one plug | Typicallyfans out to multiple LC connectors for 10-100 Gbps per connection |

SFF connectors pack more terminations into less space.

### APC vs. UPC:

These aren't connectors but polish/finish types. They affect return loss (decibel loss) from reflectance, not the core light path.

- UPC (Ultra Physical Contact): straight cut. Reflects some light into the cladding instead of the core, causing loss. Colour-coded blue.

- APC (Angled Physical Contact): angled cut that significantly reduces that loss. Colour-coded green.

### Related Bits:

- FDP (fiber distribution panel): handles fiber termination and distribution, with a cable tray and splice drawer.

- Fibre Channel (FC): a seperate high-speed (2-32 Gbps) storage transfer protocol used for SANs (Storage Area Networks). It delivers raw data losslessly, doesn't follow the OSI model, and uses different transceivers than Ethernet optical modules.

### Copper vs. Fiber:

- Distance: fiber wins for multi-mile runs. Copper tops up around 1500 ft before needing signal regeneration, and the UTP standard maxes at .

- Security: fiber wins for high-security needs, since it doesn't create a readable magnetic field.

## Twinaxial Cable & DAC

(Textbook only, not mentioned in study guide.)

- Twinaxial (twinax) cable is used for short-distance, high-speed connections like 10G/40G Ethernet in data centers. It's copper-based, so it's significally cheaper than fiber for runs of 10 meters or less.

- DAC (Direct Attach Copper) is part of the twinax family: fixed-length (~26-28 AWG, American Wire Gauge) cables with connectors on both ends for direct device-to-device communication. It uses electromagnetic shielding (like STP) to boost speed and reliability.

## Transceivers & Media Converters:

(Textbook only, not mentioned in Study Guide)

A transceiver combines a transmitter and receiver in one housing. They're used in everything from cell phones to fiber-optic systems.

| Module | What it is | Speed |
| --- | --- | --- |
| SFP (Small Form-factor Pluggable) | Compact optical transceiver module |  |
| SFP+ | Enhanced SFP | Up to 16 Gbit/s |
| QSFP (Quad SFP) | Hot-pluggable transceiver connecting network hardware to fiber or copper | 4 x 1 Gbps |
| QSFP+ | Enhanced QSFP | 4 x 10 Gbps |
| QSFP28 | Used for 100 Gbps links | 4 x 28 Gbps |

Fiber transceivers come in two flavours:

- : data flows one way only.

- Bidirectional: data flows both way simultaneously over a single strand by seperating the two devices' transmission wavelengths (per IEEE 802.3ah).

Media converters handle switching between media types:

- Single-mode fiber to Ethernet

- Multimode fiber to the Ethernet

- Fiber to coaxial

- Single-mode to multimode fiber

## Serial Cables (RS-232, DB-25, USB):

(Textbook only, not mentioned in Study Guide)

Serial communication sends one bit after another. Parallel sends bits in groups like the old Centronics printer cable, now replaced by USB.

- RS-232 (Recommended standard 232): a common serial standard connecting DTE and DCE (Data Terminal Equipment and Data Communication Equipment, like a computer to an external modem), typically via a DB-9 connector. Mostly obsolete now, replaced by USB/USB-C.

- DB-25: from the D-series connectors invented in 1952. It has 25 pins in a “B” size shell and was RS-232's usual connector. Rarely seen today. Even DB-9, once commonly used for Cisco console cables, has mostly been replaced with USB.

- USB (Universal Serial Bus):  now the standard serial bus on motherboards. Typically 4 external ports, expandable via hubs, technically supporting up to 127 devices (rarely practical). Used for printers, scanners, keyboards, mice, and more.

## Cable Properties (Speed, Distance, Duplex, Noise Immunity, Frequency):

Every cable choice trades off these five things:

| Property | What to know |
| --- | --- |
| Speed | Varies by network area. Core areas often run 10Gbps +. Distribution/access area (where users connect) typically run 100/1000 Mbps, through demand keeps pushing that up |
| Distance | All cables suffers attenuation (signal degradation over distance). Twisted-pair maxes out at 328 ft (100 m) per segment |
| Duplex | Half-duplex: can't send and receive at once, like a walkie-talkies. Full-duplex: send and receive simultaneously, doubling effective throughput. Full-duplex is standard in modern switched networks. |
| Noise immunity | Two magnetic-flux issues (below) |
| Frequency | Sets a cable's max bandwidth. Cat 5e tops out at 100 MHz (fine for 1Gbps over short runs). Cat 6 handles 250 MHz and 1 Gbps comfortably, so it's the better choice for longer runs like between floors. |

### Noise immunity, the two issues:

- Tapping: Copper's current can be “tapped” and read without even cutting the cable. STP makes this harder but not impossible. Fiber carries light, not electricity, so it's far more tap-resistant: tapping it means physically cutting and repairing the cable, which tends to get noticed.

- External EMI: Motors, speakers, and ballasts can introduce unwanted current in nearby cupper, so keep cabling away from them.

## Wiring Distribution: MDF/IDF, Blocks, Demarc:

On the building side signals pass through an MDF/IDF, terminate on a 66 or 110 block, and hand off to the provider at the demarc, the exact line between “your problem” and “their problem” when troubleshooting.

### Distribution Frames:

- MDF (Main Distribution Frame): the building's central wiring reference point and WAN termination point, where internal lines connect to external (telco) lines.

- IDF (Intermediate Distribution  Frame): connects to the MDF from an equipment/telecom room, giving more flexible distribution throughout the building.

- 25-pair cable (aka feeder cable): bundles 25 wire pairs under one jacket. Mainly for telephone backbone/cross-connecting cabling, cutting down on clutter.

### Punch-Down Blocks

| Block | What it is | Notes |
| --- | --- | --- |
| 66 block | Old analog telephone termination standard (from 1962). Uses 25-pair cable across 50 rows. | Mostly in older buildings |
| 110 block | The modern replacement Used for both phone (RJ-11) and network (RJ-45) connections. | Sizes from 25 to 500+ pairs. Supports 1 Gbps with Cat 6+ (though Cat 6's size makes a tight fit) |
| Krone | European equivalent | 110-block-compatible |
| BIX block | Another punch-down type, terminating up to 25 pairs | lip-in fitting, no pre-stripping of wires |

### The Demarc:

- Demarc (demarcation point):  the exact point where the service provider's responsibility ends. Often at the MDF, usually just an RJ-45 jack where your CSU/DSU connects to the WAN.

- Admins test both sides of the demarc to figure out if a problem is internal or the provider's fault.

- Demarc extension: the cabling between the demore and your actual office space.

- Smart jack (aka NID/NIU, Network Interface Device/Unit): a provider-owned interface, often installed because you can't directly test the bare demarc jack. It can be powered and looped for testing, and may also handle code/protocol conversion so the provider's signal works with your internal gear.

## Core Insights:

| Concept | The thing to remember |
| --- | --- |
| Coax | Braided shield resists EMI/RFI. Thinnet/thicknet (BNC) are obsolete. Survives as RG-6 /RG-59 with F-type for cable modems. |
| Plenum | Air space between floors. Fire code requires plenum-rated cable there. (non-plenum gives off poisonous smoke) |
| UTP vs. STP | Twists cut crosstalk. STP adds foil/braid for noisy environments. |
| Categories | Cat 5e = practical minimum (100 MHz, Gigabit). Cat 6 = 250 MHz. Cat 6A = 500 MHz, 10GBaseT at 100m. Cat 8 = 25G/40G at 30m. |
| Certification | Every component end-to-end must meet the category specs. |
| RJ connectors | RJ-11 = phone/DSL. RJ-45 = Ethernet. RJ-48c looks like an RJ-45, but it is for T1. |
| T568A vs. T568B | Only the orange and green pair swap. Same on both ends = straight-through. A + B = crossover. |
| Cable types | Straight-through = unlike devices. Crossover = like devices. Rollover = Cisco console, no Ethernet. |
| SMF vs. MMF | SMF = laser, long distance, between buildings. MMF = LED, cheaper, within a building. |
| Fiber connectors | ST(twist), SC (square, push-pull), LC (small, RJ-like), MT-RJ (TX/RX in one), MPO (up to 24 fibers). |
| APC vs. UPC | Polish types. APC = angled, green, less loss. UPC = straight blue. |
| Twinax / DAC | Cheap copper for ≤ 10m high-speed data center links. |
| Transceivers | SFP -> SFP + (16 Gbit/s). QSFP (4x1) -> QSFP+ (4x10) -> QSFP28 (4x28, 100G). Bidirectional = one strand, two wavelengths. |
| Serial | RS-232 with DB-9/DB-25, now replaced by USB (up to 127 devices) |
| Cable proporties | Speed, distance (UTP = 328 ft/100m), duplex, noise immunity (fiber resists tapping), frequency. |
| Wiring distribution | MDF (main, WAN termination) -> IDF. 66 block = old phone. 110 block = modern. Demarc = where the provider's responsibility ends. Smart jack = loophole test point. |

## Abbreviations:

| Abbreviation | Stands for |
| --- | --- |
| APC | Angled Physical Contact |
| AWG | American Wire Gauge |
| BIX | Building Industry Cross-connect |
| BNC | Bayonet Neill–Concelman |
| Cat | Category |
| CSU/DSU | Channel Service Unit/Data Service Unit |
| DAC | Direct Attach Copper |
| DB | D-subminiature (D-series) connector |
| DCE | Data Communication Equipment |
| DSL | Digital Subscriber Line |
| DTE | Data Terminal Equipment |
| EMI | Electromagnetic Interference |
| FC | Fibre Channel (also the FC fiber connector) |
| FDP | Fiber Distribution Panel |
| IDF | Intermediate Distribution Frame |
| IEEE | Institute of Electrical and Electronics Engineers |
| LC | Lucent Connector (local connector) |
| MDF | Main Distribution Frame |
| MMF | Multimode Fiber |
| MPO | Multi-fiber Push-On |